Name(s): _______________________________________________________________ Date: ______________

Rain Garden Lab

Rain gardens are filtering systems built to help cleanse water of excess nutrients and sediments before it flows into natural waterways.  When it rains, depending on where the water falls it may seep directly into the ground (if the ground is permeable) or it may hit buildings and roadways where it has to drain elsewhere (surfaces which are considered non-permeable).

Water may drain into catch basins, storm water drains, ditches, etc…  The water in that is draining is heading down slope.  All the water that falls on the high school grounds eventually ends up in the Sagamore Creek.  Can you think of the different means which water can utilize to move it into the Sagamore Creek?  Did you know that the Sagamore Creek leads into Great Bay:  What does Great Bay flow into?

Lately, there has been a lot of news about the water draining into our waterways in the Portsmouth area, namely Great Bay.  Fishermen and others who are familiar with the attributes of Great Bay are finding that the native organisms, such as eel grass and oysters are suffering.  The suffering is so great that there are areas of Great Bay where you cannot “fish” for oysters because they are contaminated.

Rain gardens are built by digging a depression in an area where water drains, so that the water will collect in that depression.  Then native plants are planted in the depression to help detoxify the harmful molecules present in the drainage water.  Plants are miracle workers!  They can take the toxins from the soil and reconfigure them into a form that is no longer dangerous.  Wetlands are sometimes placed in areas where water filtration is needed, such as water treatment plants.  Wetlands are full of plants that have detoxifying capabilities.  Rain gardens are made to emulate small wetlands.

In your studies of the nitrogen cycle, you have discovered that nitrogen compounds are limiting factors.  These nutrients are needed by plants in certain amounts, BUT too much nitrogen can lead to artificial eutrophication and worst – a dead zone!  Nitrogen has become notorious in the Portsmouth area of its effect on the water quality in Great Bay and the decrease in eel grass.

Before you begin, answer these questions

- What are some of the sources of nitrogen in a water body?

- What process occurs if too much nitrogen (or phosphorus) enters the water?

- What organisms are affected by nitrogen pollution?

- What do you think the connection is between a rain garden and water quality?

Materials

- 1 2-Liter empty bottle per group

- 1 Mature Rain Garden

- Optional:

  - Scissors

  - Seeds

  - Rooted spider plant

- Materials you determine would filter best

- 200mL of “polluted water”

- Water Quality Testing Station

Procedure

In this lab, you will be designing your own rain garden filtering system with the various supplies available. The purpose of this lab is to design a rain garden in hopes that it will cleanse polluted water. You will be determining the quality of the water before and after it is drained through your rain garden and compare the results to those from a mature rain garden.

- With your group, determine what materials you will use to build the rain garden. You may build something like in the picture on this page but you will want your design to be unique with the thought being that it will work the best!

- Gather the materials to build your rain garden.

*Note: You only need to do steps #3 and 4 if you do not already have a prepared bottle.

- You will need to CAREFULLY cut the 2-liter bottle in half.

- Being extremely careful, use the nail to put a hole through the cap of the bottle.

- Fill the upside down top half of the bottle with a mixture of the materials you chose will work best.

- If you want, plant no more than 2 wildflower seeds in your soil. Make sure they are spaced out and gently pressed into the soil.

- If you do not use seeds, place one of the rooted spider plants in your soil. Make sure to carefully assimilate the roots of the plant into the materials you chose.

- Once it’s built, draw and label your rain garden in the box to the right:

- For a hypothesis, discuss why you feel your design will clean the polluted water. Also include your hypothesis about the results of the water quality test for your rain garden versus the mature rain garden.

Write your hypothesis here: ___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Obtain 200 mL of “polluted water”.

- BEFORE POURING THE “POLLUTED WATER” THROUGH YOUR RAIN GARDEN, test the polluted water for all the variables in the data table below at the Water Quality Testing Station.

- Pour 100mL of the polluted water through your rain garden system (slowly!)

- Then pour the other 100mL of polluted water through the mature rain garden.

- When it is all finished draining, take the top part off each column (the rain garden section) and perform another set of water tests on both rain gardens. Record your results in the data table below.

| Water Sample | Temp (ºC) | Nitrate | pH | Phosphate | Color | Smell |
| --- | --- | --- | --- | --- | --- | --- |
| Original Polluted Water |  |  |  |  |  |  |
| YOUR RAIN GARDEN: Polluted Water After Being Filtered Through |  |  |  |  |  |  |
| MATURE RAIN GARDEN: Polluted Water After Being Filtered Through |  |  |  |  |  |  |

- Once you’ve finished the tests, clean up your work area!!! Put your rain gardens on the table by the window.

- Write a science conclusion based on your findings (remember S.C.A.R.). This may be written very neatly or typed. Staple your completed conclusion(s) to this lab.

  - Include your results and mention anything that went wrong or what you would do differently.

  - Include the comparisons between the new rain garden and the mature one.

  - Equate (compare) this lab to the actual building of a rain garden.

- Include why you think UNH is encouraging home owners and business to build rain gardens, especially if they live within the Great Bay watershed.